-NAME : - DISHA PRAJAPATI

# Assignment Module 9:

# Infrastructure Services

- Host A and Host B sit in two different subnets. The path between the subnets of these two hosts runs through three different Layer 3     forwarding devices (routers and Layer 3 switches). A network engineer uses the APIC-EM Path Trace ACL Analysis tool to analyze the path used for Host A to send packets to Host B. Which part of the function is done specifically by the ACL Analysis or ACL Trace part of the tool?

A. Discovery of the topology that exists between the two hosts

B. Analysis of the Layer 3 forwarding decisions
C. Analysis of the Layer 2 forwarding decisions
D. Analysis of the impact of ACLs on the packets

ANS :- (D)Analysis of the impact of ACLs on the packets

- ACL Analysis checks whether ACLs in the path will permit or deny the traffic between Host A and B.

- Which IPv6 address is the equivalent of the IPv4 loopback 127.0.0.1?
A. ::1
B. ::
C. 2000::/3
D. 0::/10

ANS :- (A) ::1

- ::1 is the IPv6 loopback address, same as IPv4 127.0.0.1.

- Which command is used to apply an ACL to an interface?
A. access-group
B. ip access-group
C. ip access-list
D. ip access-class
E. access-class
F. access-list

ANS :- (B) ip access-group

- ip access-group is used to apply an ACL to an interface in Cisco IOS.

- Which command and mode will successfully configure a hostname of R1?
A. Router(config)#name R1
B. Router# hostname R1
C. Router(config)#hostname R1
D. Router#name R1
E. Router>hostname R1
F. Router>name R1

ANS :- (C) Router(config)#hostname R1

- Hostname can only be set in global configuration mode using the hostname command.

- Which reserved IPv4 addresses has binary 0s in all host bit positions?
A. Local broadcast address
B. Loopback address
C. Directed broadcast address
D. Network address
E. All zeros address

ANS :- (D) Network address

- A network address has all host bits = 0 (e.g., 192.168.1.0).